How to make a simple temperature box to host your RF components for superconducting quantum computing

By Ruiqi, 28 Sep 2020

Github: https://github.com/tesla-cat/raspberry-pi-temperature-box

Part 1 Hardware

- The design follows the Optical Breadboards from Thorlabs

- First make SOLIDWORKS files like the following

- bottom.SLDPRT              2. side-short.SLDPRT

- side-long.SLDPRT

- - Then convert them into .DXF files and submit them to the mechanical workshop on level 1 of CQT

- - 1.bottom.DXF,    2.side-long.DXF,    2.side-short.DXF

- - The connection of these 3 parts are handled by standard 2020 Series Corner Brackets, I bought them from here on Amazon but the shipping took too long

- - The assembled box looks like this, with test setup

Part 2 Electronics

- The control is done by Raspberry Pi 4 Model B, it is not only used for a simple temperature control, but also chosen to handle possible future tasks, for example communicating with the Vaunix 4-8 GHz USB Programmable Signal Generator shown on the left.

- The fans operate at 12V so I made a hand soldered driver using parts found in the electronics workshop on level 2 of CQT, the schematic is

Part 3 Software

- First create 3 simple python libraries:

- temperature.py for one-wire communication with the DS18B20 temperature sensor

- pid.py for generating PID control amplitude

- pwm.py for generating the physical PWM signals sent to the MCP1407 MOSFET driver

- Then the main script is app.py

- A test is performed with 3 ZVA-183-S+ 18 GHz Amplifiers running, the following is the measured temperature (minus target value) and the PID output (divided by Kp) overtime during the test, the temperature is stabilized with < 0.1 C fluctuation